ITEM NO.    

SELF-SERVICE REFRIGERATED MERCHANDISER 

CO4778R

                  (    ) REQUIRED

A. Provide _____ (___) each self-service refrigerated merchandiser model number CO4778R as specified as manufactured by Structural Concepts.

B. Provide unit with EnergyWise Refrigeration System. Refrigeration system shall reduce daily energy consumption to a level that exceeds the Department of Energy’s 2012 requirements for open refrigerated display cases.

C. Unit to be provided with self-contained pull-out refrigeration system, condenser air intake from front and discharge from rear, integrated average product temperature of 41 degrees or less designed to operate in ambient conditions of 75 degrees F.
D. Unit to be provided with digital thermometer, black lower front panel, black ABS upper front panel, black curved header, Two laminated end panels with mirror on inside of panel, five display levels, four (4) removable shelves adjustable vertically on 1” centers, 3500k T-8 header light with protective coating, and removable deck pans to provide complete access to evaporator coil and refrigeration connections.
E. Unit to be provided with temperature terminated defrost, Clean Sweep™ automatic condenser coil cleaner, electric condensate pan, 6’ power cord, open front for grab & go selection, and ECM fan motors.

F. Voltage is to be 220/60/1. 11.47 Amps, NEMA 14-20 P.
G. Unit must comply with the following standards: ETL sanitation conforming to NSF 7, ETL Listed Certified to CAN/CSA 22.2, and to be rated in Accordance with ARI Standard 1200.  

H. Unit to be provided with one (1) year parts, labor and mileage warranty and five (5) year compressor warranty. 

I. Unit to be provided with the following accessories:
           Night curtain, pull down, non-locking
J. Unit to be delivered, uncrated, and set in place. All packing materials to be removed from job site.

