ITEM NO. 

MILK COOLER

NLFAC-8

ONE (1) REQUIRED
A.  Furnish one (1) each mobile milk cooler model no. NLFAC-8 as specified as manufactured by Delfield. Units to be 32” wide x 40” long x 42” high.
B.  Exterior top, front and sides shall be constructed of 18-gauge stainless steel. An opening shall be located in the top front of the unit to allow access to the interior. Opening shall have an “air curtain” design with cool air forced over the opening to retain the cold air in the interior of the unit. Opening shall have two (2) hinged lids to cover the top and the front. Two-piece hinged lids shall be molded of black ABS material, with an integral molded handle running the full length of each front lid. Lids shall be seated in a PVC door track.
C.  Interior liner shall be 22-gauge stainless steel (2BF finish). Cabinet shall be fully insulated with 2” of high-density foamed-in-place polyurethane.

D.  An expansion valve shall control refrigerant flow. Cabinet shall maintain 36 degrees F. to 40 degrees F. interior cabinet temperature at 100 degrees F. ambient room temperature with lids closed for storage. Properly chilled food product shall maintain 40 degrees F. for two hours with lids open.

E.  Provide .75” diameter gate valve located at the bottom of the unit.

F.  Refrigeration system shall be controlled by a timer and thermostat. Control shall be accessible through the front louver panel. Electrical connections shall be 115 volt, 60 Hertz, single phase. Unit shall have an 8’ long electrical cord and NEMA 5-15 P plug installed at the rear.
G.  Units shall hold (8) 13” x 13” x 10.50” milk crates. Compressor shall be 1/3 horsepower with condenser coil, (2) evaporator blower coils and condensate evaporator.

H.  Unit shall be mounted on 4” diameter plate casters. 

I.  Unit to be provided with the following accessories:
1. Fiberglass Inserts on Student Side and Both Ends

J.  Unit to be delivered, uncrated and set in place. All packing material to be removed from jobsite.

