ITEM NO.

RANGE MATCH CHAR BROILER

M34B

ONE (1) REQUIRED
A. Furnish one (1) each gas, range match char broiler, model number M34B as specified as manufactured by Garland.

B. Provide unit with all stainless steel front, door, front rail and sides.

C. Provide heavy-duty rack sections and high back splash with sloping sides.

D. Top racks lift off for easy cleaning.

E. Provide permanent refractory briquettes supported by high temperature steel grate rods.

F. Provide piezo spark ignition with constant burning pilots.

G. Provide large capacity grease drawers.

H. Provide storage base with interior of aluminized steel. 

I. Provide unit with 6” stainless steel adjustable legs.

J. Provide six each 15,000 BTU burners, each with Hi-Lo gas valves. 

K. Unit to be designed for ___________ gas. Control voltage is 115/60/1.

L. Provide the unit with the following accessories:

Stainless Steel Sides

Stainless Steel Legs

17” High Stainless Steel Backguard

Rear Gas Connection

M. Unit to be AGA and NSF certified.

N. Provide manufacturer's standard one (1) year parts and labor warranty.

O. Unit to be delivered, uncrated, and set in place. All packing materials to be removed from job site.

