ITEM NO.

DOUBLE CONVECTION OVEN

MCO-ES-20-S

ONE (1) REQUIRED
A. Furnish one (1) each electric, deep depth, double convection oven, model number MCO-ES-20-S as specified as manufactured by Garland.

B. Provide unit with all stainless steel front, sides, top, and legs.

C. Provide unit with porcelain oven interior, 60/40 doors with double paned thermal windows, interior lighting, and 2-speed ¾ hp fan motor.

D. Provide unit with Master 200 solid-state control panel with 150-degree F. to 500 degree F. temperature range and mechanical controls.

E. Provide “fail-safe” door that stops the fan when the oven door is opened more than 3” while the oven is operating.

F. Provide unit with 6” stainless steel legs.

G. Each oven to be provided with thirteen position rack guides and six oven racks.

H. Each oven to be rated at 11 KW for a total rating of 22 KW. 

I. Voltage to be ________ volt, 60 Hz, _____ phase..

J. Unit to be UL listed and NSF certified.

K. Provide manufacturer's two (2) year parts/labor warranty and five (5) year limited door warranty.

