ITEM NO.

DOUBLE CONVECTION OVEN

MCO-GS-20-S

ONE (1) REQUIRED
A. Furnish one (1) each gas, double convection oven, model number MCO-GS-20-S as specified as manufactured by Garland.

B. Provide unit with all stainless steel front, sides, top, and legs.

C. Provide unit with porcelain oven interior, 60/40 doors with double paned thermal windows, interior lighting, electronic spark ignition and 2-speed ¾ hp fan motor.

D. Provide unit with Master 200 solid-state control panel with 150 degree F. to 500 degree F. temperature range and mechanical controls.

E. Provide “fail-safe” door that stops the fan when the oven door is opened more than 3” while the oven is operating.

F. Provide unit with 6” stainless steel legs.

G. Each oven to be provided with thirteen position rack guides and six oven racks.

H. Each oven to be rated at 60,000 BTU/HR. 

I. Unit to be designed for ___________ gas. Control voltage is 115/60/1.

J. Unit to be UL listed and NSF certified.

K. Provide manufacturer's two (2) year parts/labor warranty and five (5) year limited door warranty.

